Osaka Sangyo University

Department of Information System Engineering

---------------------------------------------

Presenter: DANG Thanh Tin
Supervisor: Professor Seihaku Higuchi

Thesis: Compressing Medical Images and others

Starting time: October 26th 1999

Contact:

# Email: tindt@yahoo.com

# Or  sj99013@sub.osaka-sandai.ac.jp

Homepage: http://go.to/tindt

Lossless compression method used:

Coding system for long source sequences of bit-planes

REFERED FROM IEEE TRANSACTIONS ON INFORMATION THEORY, VOL. IT-27, NO.3 MAY 1981

with the following ideas:

- Gray coding

2) Gray transformation

## Refered from IEEE Transactions on Image Processing,

Vol. 4, No. 5, May 1995

##### 3) Conditional probabilities of cross points

#### 1) Gray coding

To decrease the significant impact of small gray-scale variations on the complexity of the bit planes.

If the gray levels of an m-bit gray-scale image can be represented in the form of the base-2 polynomial:

am-1 2m-1 + am-2 2m-2 +…+ a1 21 + a0 20

then the m-bit Gray code gm-1…g2g1g0 can be computed from

gi = ai ⊕ ai+1

gm-1 = am-1

And its decoding

ai = ai+1 ⊕ gi    from ai to a0

am-1 = gm-1

Here,  ⊕  denotes the exclusive OR operation.

For example:

Gray level        127          128

Binary form    01111111     10000000

(big variation)

Gray code     01000000     11000000

(small variation)

2) Gray transformation

By computing histogram, we know how many gray levels exist in an image. Suppose that the gray levels are {l0, …, lm-1}, where li < lj  if i < j, and m is the number of gray levels used in the image. Usually, m is not equal to 2K for a K-bit image. In this case, we can take a gray transform. The index n (n = 0, …, (m-1)) is used as a pseudo gray level instead of li  after canceling gray levels that have the value of zero in the histogram table. .

That means

pg (i, j) = G [g (i, j)]

= n             if g (i, j) = ln

The relation between gray levels and their indexes, i.e pseudo gray levels have to be stored for decoding.

In the detail, we estimate the Zelda image

having 120 gray levels used, for example we have the histogram table

┌─── quantity of gray level

↓

| 10 | 21 | 0 | 119 | … | 12 | 0 | … | 76 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

0     1    2     3        121  122       255

└─── gray level

After canceling the value of zero of gray level quantities, we have the relation between gray levels and pseudo gray level

┌─── gray level

↓

| 0 | 1 | 3 | …. | … | 121 | … | … | 255 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

0     1    2              78              119

└─── pseudo gray level

3) Probabilities of cross points

This graph presents the relation between pixels and their gray levels on each row of MxN image.

255

192

128

64

0

0                                 N-1

Here, we notice cross points:

- From under 128 to above 128 and inverse

- From under 64 to above 64 and inverse

- ….

Therefore, we have some ideas:

- These cross points present the transition from

bit 0 to bit 1 or inverse between 2 neighbor pixels.

- If we fold the graph by the gray levels of 128, 64, 32, … neighbor cross points are on different bit planes corresponding to the decomposition image into bit planes.

- With the folding, the probabilities of bits around cross points is much changed.

255

192

128

64

0

0                                 N-1

The first fold around the gray level of 128

In the detail, we see after folding the most significant bit plane:

- the probabilities of bit “1” of bits around the cross points is increased for current_bit_plane-1

- the probabilities of bit “0” of bits around the cross points is decreased for current_bit_plane-1

- the probabilities of bit “0” of bits around the cross points is increased for current_bit_plane-2

- the probabilities of bit “1” of bits around the cross points is decreased for current_bit_plane-2

So, we can use the information on the current bit plane (if we decompose a 256 gray-level image into 8 bit planes, from bit plane 7, …, 0), for example the bit plane 7, to encode the bit plane 6 and 5, the information on the bit plane 6 to encode the bit plane 5 and 4, ….

Therefore we define a coefficient α, it is used to define the quantity to add or to subtract to/from the probabilities and it changes from one to another plane.

Some results until now:

#### 1. Image of Zelda

Image characteristics:

Dimension: 256 x 256

Gray level: 256

### CR = 1.97501 : 1

### Remained data: 50.6327 %

2. Image of Chast

Image characteristics:

Dimension: 256 x 256

Gray level: 256

### CR = 1.87903 : 1

### Remained data: 53.2188 %

3. Image of Gazo

Image characteristics:

Dimension: 400 x 512

Gray level: 256

### CR = 2.13595 : 1

### Remained data: 46.8176 %

4. Another results

(Newest results are from web site:

http://go.to/tindt.research )

or http://come.to/tindt )

| ### Image | ### Our archieves |  | Results compared |  |
| --- | --- | --- | --- | --- |
|  | Compression Ratio | Remained data | Compression Ratio | Remained data |
| Zelda 576x720 | 1.76846:1 | 56.5463 % | 1.63:1 | 61.3496 % |
| Lena 512x512 | 1.70342:1 | 58.7056% | 1.55:1 | 64.5161 % |
| Moon 256x256 | 1.51600:1 | 65.9630 % | (no data) | (no data) |
| Girl 576x720 | 1.67505:1 | 59.6996 % | 1.59:1 | 62.8930 % |
| Mandrill 512x512 | 1.26559:1 | 79.0146 % | 1.20:1 | 83.3333 % |
| Couple 512x512 | 1.68575:1 | 59.3209 % | 1.56:1 | 64.1025 % |

Note:

-  Results compared is from the web site:

http://www.student.dtu.dk/~c971592/fagpakke.html

-  The compression ratio and the remained data are not  included the header. Header is 4274 bits.